IAEA Inter-comparison of PIGE analysis codes I: Bulk analysis

In all following exercises on bulk analysis, yield calculations will be performed with the following conditions:

- Stopping powers will be taken from SRIM-2013.

- Angular distributions will be assumed to be isotropic.

- Absorption of gamma-rays will not be considered.

- Smooth cross sections known down to the threshold energy

a) A target with only one element of a smooth cross section (without sharp resonances) and an energy step of around 20 keV that is measured down to the threshold energy, for example the 718 keV gamma ray of 10B(p,p’g) 10B

| Beam Energy (keV) | Egamma (keV) | Theta | Target | Cross Section |
| --- | --- | --- | --- | --- |
| 5000-3000 200 keV step | 718 | 165 | B 100% | A. Lagoyannis et al., NIMB 342 (2015) 271-276 |

b) The same element but in a matrix of three elements (B-O-Fe) where the concentration of Fe is known from some other experiment.

| Beam Energy (keV) | Egamma (keV) | Theta | Target | Cross Section |
| --- | --- | --- | --- | --- |
| 5000-3000 200 keV step | 718 | 165 | B 10% O 40% Fe 50% | A. Lagoyannis et al., NIMB 342 (2015) 271-276 |

- Smooth cross section known down to an energy

Cases 1a and 1b can be implemented for a smooth cross section that is known down to an energy and an initial yield exists from that energy down to the threshold energy (the same cross sections as cases 1 and 2 can be used but without using the whole cross section but only a part of it) or the 10B(p,ag) 7Be reaction

10B(p,ag) 7Be reaction  :

| Beam Energy (keV) | Egamma (keV) | Theta | Target | Cross Section |
| --- | --- | --- | --- | --- |
| 3800-3000 100 keV step | 429 | 150 | B 100% | A. Lagoyannis et al., NIMB 342 (2015) 271-276 |

10B(p,p’g) 10B  :

| Beam Energy (keV) | Egamma (keV) | Theta | Target | Cross Section |
| --- | --- | --- | --- | --- |
| 3800-3000 100 keV step | 718 | 150 | B 100% | A. Lagoyannis et al., NIMB 342 (2015) 271-276 |

This dataset has also a small gap in energies. It would be interesting to see how the codes perform

- Cross section with sharp resonance

- A target with only one element with sharp resonances for example the 197 keV gamma ray from the 19F(p,pg2)19F reaction.

| Beam Energy (keV) | Egamma (keV) | Theta | Target | Cross Section |
| --- | --- | --- | --- | --- |
| 2700-1000 5 keV step | 197 | 130 | F 100% | A.P. Jesus et al., Nucl. Instr. Meth. B 161-163 (2000) 186 |

- The same as a) but with 3 elements in the matrix

| Beam Energy (keV) | Egamma (keV) | Theta | Target | Cross Section |
| --- | --- | --- | --- | --- |
| 2700-1000 5 keV step | 197 | 130 | F 10% O 30% Cr 60% | A.P. Jesus et al., Nucl. Instr. Meth. B 161-163 (2000) 186 |

4.	 Add straggling calculations to cases 1 to 3 using Bohr’s model with a beam ripple of 3 keV.